Documentaie

Tema: model urdf animat de un node

Nume: Peteu Mircea

Cap 1, Modelul

Comenzi folosite:

$ cd ../.. (navigare in terminal, in cazul asta desktop)

$ touch –filename.extension- (creat un file cu numele “robot_T2.urdf”)

$ touch –x –filename.extension- (declaram ca este executabila file-ul creat)

$ code –filename.extension- (open –filename.extension- in vs code to edit, *urdf support addon* utilizat la editare)

Proces:

Utilizat tutorialul pentru modelare: https://www.youtube.com/watch?v=dZ_CyyEvBE0

Comanda pentrul rulare in rviz/editare cu rviz:

$ ros2 launch urdf_tutorial display.launch.py model:=/home/thor/Desktop/robot_T2.urdf

Code:

<?xml version="1.0"?>

<robot name="robot_T2">

<!-- Materials -->

<material name="gray">

<color rgba="0.7 0.7 0.7 1.0"/>

</material>

<material name="green">

<color rgba="0.0 0.6 0.0 1.0"/>

</material>

<material name="white">

<color rgba="1.0 1.0 1.0 1.0"/>

</material>

<!-- Tractor Body -->

<link name="base_link">

<visual>

<geometry>

<box size="1.2 0.7 0.4"/> <!-- Larger, more tractor-like body -->

</geometry>

<origin xyz="0 0 0.4" rpy="0 0 0"/>

<material name="green"/>

</visual>

</link>

<!-- Lidar -->

<link name="lidar_link">

<visual>

<geometry>

<cylinder radius="0.1" length="0.05"/>

</geometry>

<origin xyz="0.0 0.0 0.0" rpy="0.0 0.0 0.0"/>

<material name="white"/>

</visual>

</link>

<link name="front_left_wheel">

<visual>

<geometry>

<cylinder radius="0.2" length="0.1"/>

</geometry>

<origin xyz="0.0 0.0 0.0" rpy="0 0 0"/>

<material name="gray"/>

</visual>

<visual>

<geometry>

<box size="0.3 0.05 0.11"/>

</geometry>

<origin xyz="0.0 0.0 0.0" rpy="0 0 0"/>

<material name="white"/>

</visual>

</link>

<link name="front_right_wheel">

<visual>

<geometry>

<cylinder radius="0.2" length="0.1"/>

</geometry>

<origin xyz="0.0 0.0 0.0" rpy="0 0 0"/>

<material name="gray"/>

</visual>

<visual>

<geometry>

<box size="0.3 0.05 0.11"/>

</geometry>

<origin xyz="0.0 0.0 0.0" rpy="0 0 0"/>

<material name="white"/>

</visual>

</link>

<link name="back_left_wheel">

<visual>

<geometry>

<cylinder radius="0.4" length="0.1"/>

</geometry>

<origin xyz="0.0 0.0 0.0" rpy="0 0 0"/>

<material name="gray"/>

</visual>

<visual>

<geometry>

<box size="0.6 0.1 0.11"/>

</geometry>

<origin xyz="0.0 0.0 0.0" rpy="0 0 0"/>

<material name="white"/>

</visual>

</link>

<link name="back_right_wheel">

<visual>

<geometry>

<cylinder radius="0.4" length="0.1"/>

</geometry>

<origin xyz="0.0 0.0 0.0" rpy="0 0 0"/>

<material name="gray"/>

</visual>

<visual>

<geometry>

<box size="0.6 0.1 0.11"/>

</geometry>

<origin xyz="0.0 0.0 0.0" rpy="0 0 0"/>

<material name="white"/>

</visual>

</link>

<!-- Exhaust Pipe Left -->

<link name="Ex_pipe_left">

<visual>

<geometry>

<cylinder radius="0.05" length="0.7"/>

</geometry>

<origin xyz="0.0 0.0 0.0" rpy="0 0 0"/>

<material name="white"/>

</visual>

</link>

<!-- Exhaust Pipe Right -->

<link name="Ex_pipe_right">

<visual>

<geometry>

<cylinder radius="0.05" length="0.7"/>

</geometry>

<origin xyz="0.0 0.0 0.0" rpy="0 0 0"/>

<material name="white"/>

</visual>

</link>

<!-- End Pipe Left -->

<link name="End_pipe_left">

<visual>

<geometry>

<cylinder radius="0.05" length="0.2"/>

</geometry>

<origin xyz="0.0 0.0 0.0" rpy="0 0 0"/>

<material name="white"/>

</visual>

</link>

<!-- End Pipe Right -->

<link name="End_pipe_right">

<visual>

<geometry>

<cylinder radius="0.05" length="0.2"/>

</geometry>

<origin xyz="0.0 0.0 0.0" rpy="0 0 0"/>

<material name="white"/>

</visual>

</link>

<!-- Headlight Left -->

<link name="headlight_left">

<visual>

<geometry>

<cylinder radius="0.05" length="0.01"/>

</geometry>

<origin xyz="0.0 0.0 0.0" rpy="0 0 0"/>

<material name="white"/>

</visual>

</link>

<!-- Headlight Right -->

<link name="headlight_right">

<visual>

<geometry>

<cylinder radius="0.05" length="0.01"/>

</geometry>

<origin xyz="0.0 0.0 0.0" rpy="0 0 0"/>

<material name="white"/>

</visual>

</link>

<!-- Joints -->

<!-- Base to Lidar -->

<joint name="base_lidar" type="fixed">

<parent link="base_link"/>

<child link="lidar_link"/>

<origin xyz="0.0 0.0 0.6" rpy="0.0 0.0 0.0"/>

</joint>

<!-- Base to Front Left Wheel -->

<joint name="base_front_left_wheel" type="continuous">

<parent link="base_link"/>

<child link="front_left_wheel"/>

<origin xyz="-0.4 0.4 0.2" rpy="1.57 0.0 0.0"/>

<axis xyz="0 0 1"/>

</joint>

<!-- Base to Front Right Wheel -->

<joint name="base_front_right_wheel" type="continuous">

<parent link="base_link"/>

<child link="front_right_wheel"/>

<origin xyz="-0.4 -0.4 0.2" rpy="1.57 0.0 0.0"/>

<axis xyz="0 0 1"/>

</joint>

<!-- Base to Back Left Wheel -->

<joint name="base_back_left_wheel" type="continuous">

<parent link="base_link"/>

<child link="back_left_wheel"/>

<origin xyz="0.5 0.4 0.4" rpy="1.57 0.0 0.0"/>

<axis xyz="0 0 1"/>

</joint>

<!-- Base to Back Right Wheel -->

<joint name="base_back_right_wheel" type="continuous">

<parent link="base_link"/>

<child link="back_right_wheel"/>

<origin xyz="0.5 -0.4 0.4" rpy="1.57 0.0 0.0"/>

<axis xyz="0 0 1"/>

</joint>

<!-- Base to Exhaust Pipe Left -->

<joint name="base_ex_pipe_left" type="fixed">

<parent link="base_link"/>

<child link="Ex_pipe_left"/>

<origin xyz="0.65 0.25 0.55" rpy="0 0 0"/>

</joint>

<!-- Base to Exhaust Pipe Right -->

<joint name="base_ex_pipe_right" type="fixed">

<parent link="base_link"/>

<child link="Ex_pipe_right"/>

<origin xyz="0.65 -0.25 0.55" rpy="0 0 0"/>

</joint>

<!-- Exhaust Pipe Left to End Pipe Left -->

<joint name="ex_pipe_left_end_pipe_left" type="fixed">

<parent link="Ex_pipe_left"/>

<child link="End_pipe_left"/>

<origin xyz="0.05 0.0 0.4" rpy="0 1.57 0"/>

</joint>

<!-- Exhaust Pipe Right to End Pipe Right -->

<joint name="ex_pipe_right_end_pipe_right" type="fixed">

<parent link="Ex_pipe_right"/>

<child link="End_pipe_right"/>

<origin xyz="0.05 0.0 0.4" rpy="0 1.57 0"/>

</joint>

<!-- Base to Headlight Left -->

<joint name="base_headlight_left" type="fixed">

<parent link="base_link"/>

<child link="headlight_left"/>

<origin xyz="-0.6 0.25 0.3" rpy="0 1.57 0"/>

</joint>

<!-- Base to Headlight Right -->

<joint name="base_headlight_right" type="fixed">

<parent link="base_link"/>

<child link="headlight_right"/>

<origin xyz="-0.6 -0.25 0.3" rpy="0 1.57 0"/>

</joint>

</robot>

Rezultat code:

Salvare:

Save as-> .rviz -> robot.rviz (cum am salvat eu, cel putin; si se pastreaza pentru mai incolo, preferabil pe descktop)

Cap 2, Packetul

Revenim la pachetul creat anterior, si creem un node nou de folosit:

$ cd ../.. (pentru a intra in ros2_ws, src, tema1, tema1; unde tinem nodurile)

$ touch wheel_movement_node.py (creem un node nou)

$ chmod –x wheel_movement_node.py (o facem executabila)

$ code wheel_movement_node.py (sa editam cu urmatorul code)

import rclpy

from rclpy.node import Node

from sensor_msgs.msg import JointState

import math

import threading

import time

class WheelMovementNode(Node):

def __init__(self):

super().__init__('wheel_movement_node')

self.joint_pub = self.create_publisher(JointState, '/joint_states', 10)

# Variables for animation

self.current_angle = 0.0

self.target_angle = 0.0

self.animation_speed = 0.1  # Radians per tick

# Run the publisher loop at 30Hz for smooth animation

self.timer = self.create_timer(0.033, self.update_and_publish)

self.get_logger().info("Wheel node ready.")

self.get_logger().info("[Exemple: (Correct:5); (Incorrect:5m)]")

self.get_logger().info("Type a distance and hit Enter:")

# Input thread

self.input_thread = threading.Thread(target=self.input_loop)

self.input_thread.daemon = True #tl;dr: the daemon is used to promptly retrive node and topic information. <-from github:

#https://github.com/clearpathrobotics/roscon2024-workshop-demystifying-ros2-networking/blob/main/exercises/2_daemon.md

self.input_thread.start()

def input_loop(self):

while rclpy.ok():

try:

user_input = input("Target Distance: ")

distance = float(user_input)

# Calculate the target angle

wheel_circumference = 1.0 #perimetru rotilor

total_rotation = (distance / wheel_circumference) * 2 * math.pi

self.target_angle = total_rotation # *180/pi pt deg

self.get_logger().info(f"Moving to angle: {self.target_angle:.2f} rad")

except ValueError:

print("Incorrect.")

def update_and_publish(self):

# -- Animation Logic --

# Slowly move current_angle towards target_angle

if abs(self.current_angle - self.target_angle) > 0.01: #abs = Returns the absolute value of a number. Ex: |-3| = 3

if self.current_angle < self.target_angle:         # Ex: |0 (original) - n|=m; m<n (unghi dorit)=> misca inainte

self.current_angle += self.animation_speed     #                           m<n (unghi dorit)=> misca inapoi

else:

self.current_angle -= self.animation_speed

else:

# Snap to exact value if very close

self.current_angle = self.target_angle

# -- Publish --

# Create a new JointState message object

msg = JointState()

# Add a timestamp. This is CRITICAL for RViz/TF.

# It tells the system "this is where the joints are AT THIS MOMENT."

msg.header.stamp = self.get_clock().now().to_msg()

# List the names of the joints exactly as they appear in your URDF.

msg.name = [

'base_front_left_wheel',

'base_front_right_wheel',

'base_back_left_wheel',

'base_back_right_wheel'

]

# Assign the current rotation (in radians) to each joint.

msg.position = [

self.current_angle,

self.current_angle,

self.current_angle,

self.current_angle

]

# Send the message to the /joint_states topic.

# Robot_state_publisher hears this and updates the 3D model.

self.joint_pub.publish(msg)

def main(args=None):

rclpy.init(args=args)

node = WheelMovementNode()

try:

rclpy.spin(node)

except KeyboardInterrupt:

pass

finally:

node.destroy_node()

rclpy.shutdown()

if __name__ == '__main__':

main()

Editam apoi setup.py creat anterior:

data_files=[

('share/ament_index/resource_index/packages',

['resource/' + package_name]),

('share/' + package_name, ['package.xml']),

('share/' + package_name + '/launch', ['robot.launch.py']),

('share/' + package_name + '/rviz', ['robot.rviz']),

],

-- and --

entry_points={

'console_scripts': ['intsqare = tema1.intsqare:main',

'client = tema1.client:main',

'wheel_movement = tema1.wheel_movement_node:main',

],

Apoi folosim $touch si $chmod din nous a facem un robot.launch.py in tema1 (primul nu cel cu nodurile) si intrducem codul urmator:

import os

from launch import LaunchDescription

from launch_ros.actions import Node

def generate_launch_description():

urdf_file = os.path.join(

os.getenv('HOME'), 'Desktop', 'robot_T2.urdf'

)

with open(urdf_file, 'r') as f:

robot_description = f.read()

rviz_config = os.path.join(

os.getenv('HOME'), 'ros2_ws', 'src', 'tema1', 'robot.rviz'

)

return LaunchDescription([

# 1. Publishes the robot structure (URDF) to TF

Node(

package='robot_state_publisher',

executable='robot_state_publisher',

output='screen',

parameters=[{'robot_description': robot_description}],

),

# 2. Opens RViz

Node(

package='rviz2',

executable='rviz2',

arguments=['-d', rviz_config],

output='screen',

),

])

Ne trebuie ambele (urdf si .rviz), urdf e schita, practice modelul in sine si .rviz exte cel care pune/animeaza [ps: sa se modifice path-urile daca sunt in alte parti puse decat la mine]

Acum punem .rviz in pachet sau unde vrem (atata timp cat avem conectat path-ul correct in launch…)

Si dam $ colcon build in ros2_ws, apoi $ source /opt/user/distributie/setup.bash si folosim urmatoarele comenzi in terminale diferite (dup ace dam source in fiecare):

$ ros2 run tema1 wheel_movement

$ ros2 launch tema1 robot.launch.py